�����������������������������������������������������������������������������

� GUSDOC �

����������

THE OFFICAL

GRAVIS ULTRASOUND PROGRAMMERS ENCYCLOPEDIA

( G.U.P.E )

v 0.1

Written by Mark Dixon.

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

INTRODUCTION

~~~~~~~~~~~~

The Gravis Ultrasound is by far the best & easiest sound card to

program. Why? Because the card does all the hard stuff for you, leaving

you and the CPU to do other things! This reference will document some

(but not all) of the Gravis Ultrasound's hardware functions, allowing

you to play music & sound effects on your GUS.

We will not be going into great detail as to the theory behind

everything - if you want to get technical information then read the

GUS SDK. We will be merely providing you with the routines necessary

to play samples on the GUS, and a basic explanation of how they work.

This document will NOT go into DMA transfer or MIDI specifications.

If someone knows something about them, and would like to write some

info on them, we would appreciate it very much.

All source code is in Pascal (tested under Turbo Pascal v7.0, but

should work with TP 6.0 and possibly older versions). This document

will assume reasonable knowledge of programming, and some knowledge of

soundcards & music.

INITIALISATION & AUTODETECTION

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Since we are not using DMA, we only need to find the GUS's I/O port,

which can be done from the DOS environment space, or preferably from a

routine that will scan all possible I/O ports until it finds a GUS.

The theory behind the detection routine is to store some values into

GUS memory, and then read them back. If we have the I/O port correct,

we will read back exactly what we wrote. So first, we need a routine

that will write data to the memory of the GUS :

Function  GUSPeek(Loc : Longint) : Byte;

{ Read a value from GUS memory }

Var

B : Byte;

AddLo : Word;

AddHi : Byte;

Begin

AddLo := Loc AND $FFFF;

AddHi := LongInt(Loc AND $FF0000) SHR 16;

Port [Base+$103] := $43;

Portw[Base+$104] := AddLo;

Port [Base+$103] := $44;

Port [Base+$105] := AddHi;

B := Port[Base+$107];

GUSPeek := B;

End;

Procedure GUSPoke(Loc : Longint; B : Byte);

{ Write a value into GUS memory }

Var

AddLo : Word;

AddHi : Byte;

Begin

AddLo := Loc AND $FFFF;

AddHi := LongInt(Loc AND $FF0000) SHR 16;

Port [Base+$103] := $43;

Portw[Base+$104] := AddLo;

Port [Base+$103] := $44;

Port [Base+$105] := AddHi;

Port [Base+$107] := B;

End;

Since the GUS can have up to 1meg of memory, we need to use a 32bit

word to address all possible memory locations. However, the hardware of

the GUS will only accept a 24bit word, which means we have to change

the 32bit address into a 24bit address. The first two lines of each

procedure does exactly that.

The rest of the procedures simply send commands and data out through

the GUS I/O port defined by the variable BASE (A word). So to test for

the presence of the GUS, we simply write a routine to read/write memory

for all possible values of BASE :

Function GUSProbe : Boolean;

{ Returns TRUE if there is a GUS at I/O address BASE }

Var

B : Byte;

Begin

Port [Base+$103] := $4C;

Port [Base+$105] := 0;

GUSDelay;

GUSDelay;

Port [Base+$103] := $4C;

Port [Base+$105] := 1;

GUSPoke(0, $AA);

GUSPoke($100, $55);

B := GUSPeek(0);

If B = $AA then GUSProbe := True else GUSProbe := False;

End;

Procedure GUSFind;

{ Search all possible I/O addresses for the GUS }

Var

I : Word;

Begin

for I := 1 to 8 do

Begin

Base := $200 + I*$10;

If GUSProbe then I := 8;

End;

If Base < $280 then

Write('Found your GUS at ', Base, ' ');

End;

The above routines will obviously need to be customised for your own

use - for example, setting a boolean flag to TRUE if you find a GUS,

rather than just displaying a message.

It is also a good idea to find out exactly how much RAM is on the

GUS, and this can be done in a similar process to the above routine.

Since the memory can either be 256k, 512k, 768k or 1024k, all we have

to do is to read/write values on the boundaries of these memory

addresses. If we read the same value as we wrote, then we know exactly

how much memory is available.

Function  GUSFindMem : Longint;

{ Returns how much RAM is available on the GUS }

Var

I : Longint;

B : Byte;

Begin

GUSPoke($40000, $AA);

If GUSPeek($40000) <> $AA then I := $3FFFF

else

Begin

GUSPoke($80000, $AA);

If GUSPeek($80000) <> $AA then I := $8FFFF

else

Begin

GUSPoke($C0000, $AA);

If GUSPeek($C0000) <> $AA then I := $CFFFF

else I := $FFFFF;

End;

End;

GUSFindMem := I;

End;

Now that we know where the GUS is, and how much memory it has, we

need to initialise it for output. Unfortunately, the below routine is

slightly buggy. If you run certain programs (I discovered this after

running Second Reality demo) that use the GUS, and then your program

using this init routine, it will not initialise the GUS correctly.

It appears that I am not doing everything that is necessary to

initialise the GUS. However, I managed to correct the problem by

either re-booting (not a brilliant solution) or running Dual Module

Player, which seems to initialise it properly. If someone knows where

i'm going wrong, please say so!

Anyway, the following routine should be called after you have found

the GUS, and before you start doing anything else with the GUS.

Procedure GUSDelay; Assembler;

{ Pause for approx. 7 cycles. }

ASM

mov   dx, 0300h

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

End;

Procedure GUSReset;

{ An incomplete routine to initialise the GUS for output. }

Begin

port [Base+$103]   := $4C;

port [Base+$105] := 1;

GUSDelay;

port [Base+$103]   := $4C;

port [Base+$105] := 7;

port [Base+$103]   := $0E;

port [Base+$105] := (14 OR $0C0);

End;

Now you have all the routine necessary to find and initialise the

GUS, let's see just what we can get the GUS to do!

MAKING SOUNDS

~~~~~~~~~~~~~

The GUS is unique in that it allows you to store the data to be

played in it's onboard DRAM. To play the sample, you then tell it what

frequency to play it at, what volume and pan position, and which sample

to play. The GUS will then do everything in the background, it will

interpolate the data to give an effective 44khz (or less, depending on

how many active voices) sample. This means that an 8khz sample will

sound better on the GUS than most other cards, since the GUS will play

it at 44khz!

The GUS also has 32 seperate digital channels (that are mixed by a

processor on the GUS) which all have their own individual samples,

frequencies, volumes and panning positions. For some reason, however,

the GUS can only maintain 44khz output with 16 channels - the more

channels, the lower the playback rate (which basically means, lower

quality). If you are using all 32 channels (unlikely), then playback is

reduced to 22khz.

Since you allready know how to store samples in the GUS dram (simply

use the GUSPoke routine to store the bytes) we will now look at various

routines to change the way the gus plays a sample. The first routine we

will look at will set the volume of an individual channel :

Procedure GUSSetVolume( Voi : Byte; Vol : Word);

{ Set the volume of channel VOI to Vol, a 16bit logarithmic scale

volume value -  0 is off, $ffff is full volume, $e0000 is half

volume, etc }

Begin

Port [Base+$102] := Voi;

Port [Base+$102] := Voi;

Port [Base+$102] := Voi;

Port [Base+$103] := 9;

Portw[Base+$104] := Vol;  { 0-0ffffh, log scale not linear }

End;

The volume (and pan position & frequency) can be changed at ANY time

regardless of weather the GUS is allready playing the sample or not.

This means that to fade out a sample, you simply make several calls to

the GUSSetVolume routine with exponentially (to account for the

logarithmic scale) decreasing values.

The next two routines will set the pan position (from 0 to 15, 0

being left, 15 right and 7 middle) and the frequency respectively :

Procedure GUSSetBalance( V, B : Byte);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $C;

Port [Base+$105] := B;

End;

Procedure GUSSetFreq( V : Byte; F : Word);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := 1;

Portw[Base+$104] := F;

End;

I'm not sure the the value F in the set frequency procedure. The GUS

SDK claims that it is the exact frequency at which the sample should be

played.

When playing a sample, it is necessary to set the volume, position

and frequency BEFORE playing the sample. In order to start playing a

sample, you need to tell the GUS where abouts in memory the sample is

stored, and how big the sample is  :

Procedure GUSPlayVoice( V, Mode : Byte;VBegin, VStart, VEnd : Longint);

{ This routine tells the GUS to play a sample commencing at VBegin,

starting at location VStart, and stopping at VEnd }

Var

GUS_Register : Word;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $0A;

Portw[Base+$104] := (VBegin SHR 7) AND 8191;

Port [Base+$103] := $0B;

Portw[Base+$104] := (VBegin AND $127) SHL 8;

Port [Base+$103] := $02;

Portw[Base+$104] := (VStart SHR 7) AND 8191;

Port [Base+$103] := $03;

Portw[Base+$104] := (VStart AND $127) SHL 8;

Port [Base+$103] := $04;

Portw[Base+$104] := ((VEnd)   SHR 7) AND 8191;

Port [Base+$103] := $05;

Portw[Base+$104] := ((VEnd)   AND $127) SHL 8;

Port [Base+$103] := $0;

Port [Base+$105] := Mode;

{ The below part isn't mentioned as necessary, but the card won't

play anything without it! }

Port[Base] := 1;

Port[Base+$103] := $4C;

Port[Base+$105] := 3;

end;

There are a few important things to note about this routine. Firstly,

the value VEnd refers to the location in memory, not the length of the

sample. So if the sample commenced at location 1000, and was 5000 bytes

long, the VEnd would be 6000 if you wanted the sample to play to the

end. VBegin and VStart are two weird values, one of them defines the

start of the sample, and the other defines where abouts to actually

start playing. I'm not sure why both are needed, since I have allways

set them to the same value.

Now that the gus is buisy playing a sample, the CPU is totally free

to be doing other things. We might, for example, want to spy on the gus

and see where it is currently up to in playing the sample :

Function VoicePos( V : Byte) : Longint;

Var

P : Longint;

Temp0, Temp1 : Word;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $8A;

Temp0 := Portw[Base+$104];

Port [Base+$103] := $8B;

Temp1 := Portw[Base+$104];

VoicePos := (Temp0 SHL 7)+ (Temp1 SHR 8);

End;

This routine will return the memory location that the channel V is

currently playing. If the GUS has reached the end of the sample, then

the returned value will be VEnd. If you want to see what BYTE value is

currently being played (for visual output of the sample's waveform),

then you simply PEEK the location pointed to by this routine.

Finally, we might want to stop playing the sample before it has

reached it's end - the following routine will halt the playback on

channel V.

Procedure GUSStopVoice( V : Byte);

Var

Temp : Byte;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $80;

Temp := Port[Base+$105];

Port [Base+$103] := 0;

Port [Base+$105] := (Temp AND $df) OR 3;

GUSDelay;

Port [Base+$103] := 0;

Port [Base+$105] := (Temp AND $df) OR 3;

End;

SPECIAL EFFECTS

~~~~~~~~~~~~~~~

There are a few extra features of the GUS that are worthy of mention,

the main one being hardware controlled sample looping. The GUS has a

control byte for each of the 32 channels. This control byte consists of

8 flags that effect the way the sample is played, as follows :

( The table is taken directly from the GUS Software Developers Kit )

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Voice Stopped

|   |   |   |   |   |   +-------- Stop Voice

|   |   |   |   |   +------------ 16 bit data

|   |   |   |   +---------------- Loop enable

|   |   |   +-------------------- Bi-directional loop enable

|   |   +------------------------ Wave table IRQ

|   +---------------------------- Direction of movement

+-------------------------------- IRQ pending

(*)Bit 0 = 1 : Voice is stopped. This gets set by hitting the end

address (not looping) or by setting bit 1 in this reg.

Bit 1 = 1 : Stop Voice. Manually force voice to stop.

Bit 2 = 1 : 16 bit wave data, 0 = 8 bit data

Bit 3 = 1 : Loop to begin address when it hits the end address.

Bit 4 = 1 : Bi-directional looping enabled

Bit 5 = 1 : Enable wavetable IRQ. Generate an irq when the voice

hits the end address. Will generate irq even if looping

is enabled.

(*)Bit 6 = 1 - Decreasing addresses, 0 = increasing addresses. It is

self-modifying because it might shift directions when

it hits one of the loop boundaries and looping is enabled.

(*)Bit 7 = 1 - Wavetable IRQ pending. If IRQ's are enabled and

looping is NOT enabled, an IRQ will be constantly

generated until voice is stopped. This means that

you may get more than 1 IRQ if it isn't handled

properly.

Procedure GUSVoiceControl( V, B : Byte);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $0;

Port [Base+$105] := B;

End;

The above routine will set the Voice Control byte for the channel

defined in V. For example, if you want channel 1 to play the sample in

a continuous loop, you would use the procedure like this :

GUSVoiceControl( 1, $F );  { Bit 3 ON = $F }

CONCLUSION

~~~~~~~~~~

The above routines are all that is necessary to get the GUS to start

playing music. To prove this, I have included my 669 player & source

code in the package as a practical example. The GUSUnit contains all

the routines discussed above. I won't go into the theory of the 669

player, but it is a good starting point if you want to learn about

modplayers. The player is contained within the archive 669UNIT.ARJ

�����������������������������������������������������������������������������

� README �

����������

GUS669 Unit  v0.2b

Copyright 1994 Mark Dixon.

(aka C.D. of Silicon Logic)

LEGAL STUFF

~~~~~~~~~~~

I'd like to avoid this, but it has to be done. Basically, if anything

in this archive causes any kind of damage, I cannot be held

responsable - USE AT YOUR OWN RISK.

In adition, since I spent long hours working on this project, and

attempting to decode the GUS SDK, I would appreciate it if people

didn't rip off my work. Give me credit for what I have done, and if

your planning to use my routines for commercial purposes, talk to me

first, or you might find yourself on the wrong side of a legal battle.

(Hey, let's sound tough while i'm at it, I have lawyer's in the

family, so it's not gonna cost me much to sue someone. And don't

criticise my spelling! :)

BORING STUFF

~~~~~~~~~~~~

Well, if your the sort of person who likes to ignore all the rubishy

bits that go into a README text file, then you'd better stop now and

go and try out the source code!

Basically, this readme isn't going to say much more than what the

source code is, and then go dribling on for five pages about

absolutely nothing.

SOURCE CODE! DID SOMEONE SAY - SOURCE CODE!! - ????

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Yes, that's right, free with every download of this wonderful archive

comes the complete Pascal source code to a 669 module player for the

GUS. I'd have included my MOD player, but I haven't been able to get

all the MOD commands working, so you'll just have to make do with a

669 player :)

Feel free to make use of this source code for any non-commercial

purposes you might be able to think of - and mention my name while

your at it! Since the source code is here, people are bound to modify

it for their personal uses. If you do this, I would very much like to

see your modifications - so that I can include them in the next

release of the player.

Well, I don't want to bore you anymore, and it's getting late (not!)

so i'd better let you go and play around with the source code :)

SILICON LOGIC

~~~~~~~~~~~~~

What ever happened to Silicon Logic? Well, after being killed off over

in Perth, a major revival is underway here in Canberra, with a more

commercial view - more on that later.

For those of you who have never heard of Silicon Logic, then you're

either not Australian, or not into the ausie demo scene. But then,

that covers about 99.999999999999% of the world population :)

GREETINGS

~~~~~~~~~

I've allways wanted to dribble some thanks, so here goes.

Thanks go to...

Darren Lyon    - Who got me into this programming lark in the first

place. Finally wrote myself a mod player :)

Tran           - Your source code really helped!

Kitsune        - Love those mods, keep up the good work!

... and Advanced Gravis, for making the best sound card ever.

Greetings to...

FiRE members   - I'll probably never join you guys, but good luck

anyway!

UNiQUE         - How's the board going?

CRaSH          - Still ripping other peoples source code?

Old SL members - Thanks for the support, good luck with your new

group!

Oliver White   - G'day... just thought i'd say hi, since you so

kindly beta tested the player for me.

Murray Head    - Rick Price sux! :-) SoundBlaster sux too! :-)

Perth people   - I'm coming back... someday!

THE PICK / MINNOW   -  Hey, give me a call sometime, long time no

talk...

INTERESTED IN A DEMO GROUP IN CANBERRA?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

If there is anyone interested in joining a demo / coding group in

Canberra (ACT), then drop me a line.

�����������������������������������������������������������������������������

� GUSUNIT.PAS�

��������������

Unit  GUSUnit;

{

GUS DigiUnit  v1.0

Copyright 1994 Mark Dixon.

This product is "Learnware".

All contents of this archive, including source and executables, are the

intellectual property of the author, Mark Dixon. Use of this product for

commercial programs, or commercial gain in ANY way, is illegal. Private

use, or non-commercial use (such as demos, PD games, etc) is allowed,

provided you give credit to the author for these routines.

Feel free to make any modifications to these routines, but I would

appreciate it if you sent me these modifications, so that I can include

them in the next version of the Gus669 Unit.

If you wish to use these routines for commercial purposes, then you will

need a special agreement. Please contact me, Mark Dixon, and we can work

something out.

What's "Learnware"? Well, I think I just made it up actually. What i'm

getting at is that the source code is provided for LEARNING purposes only.

I'd get really angry if someone ripped off my work and tried to make out

that they wrote a mod player.

As of this release (Gus699 Unit), the Gus DigiUnit has moved to version

1.0, and left the beta stage. I feel these routines are fairly sound,

and I haven't made any changes to them in weeks.

Notice the complete absence of comments here? Well, that's partially

the fault of Gravis and their SDK, since it was so hard to follow, I

was more worried about getting it working than commenting it. No offense

to Gravis though, since they created this wonderful card! :-) It helps

a lot if you have the SDK as a reference when you read this code,

otherwise you might as well not bother reading it.

}

INTERFACE

Procedure GUSPoke(Loc : Longint; B : Byte);

Function  GUSPeek(Loc : Longint) : Byte;

Procedure GUSSetFreq( V : Byte; F : Word);

Procedure GUSSetBalance( V, B : Byte);

Procedure GUSSetVolume( Voi : Byte; Vol : Word);

Procedure GUSPlayVoice( V, Mode : Byte;VBegin, VStart, VEnd : Longint);

Procedure GUSVoiceControl( V, B : Byte);

Procedure GUSReset;

Function VoicePos( V : Byte) : Longint;

Const

Base : Word = $200;

Mode : Byte = 0;

IMPLEMENTATION

Uses Crt;

Function Hex( W : Word) : String;

Var

I, J : Word;

S : String;

C : Char;

Const

H : Array[0..15] of Char = ('0','1','2','3','4','5','6','7','8','9','A','B','C','D','E','F');

Begin

S := '';

S := S + H[(W DIV $1000) MOD 16];

S := S + H[(W DIV $100 ) MOD 16];

S := S + H[(W DIV $10  ) MOD 16];

S := S + H[(W DIV $1   ) MOD 16];

Hex := S+'h';

End;

Procedure GUSDelay; Assembler;

ASM

mov   dx, 0300h

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

in    al, dx

End;

Function VoicePos( V : Byte) : Longint;

Var

P : Longint;

I, Temp0, Temp1 : Word;

Begin

Port [Base+$102] := V;

Port [Base+$103] := $8A;

Temp0 := Portw[Base+$104];

Port [Base+$103] := $8B;

Temp1 := Portw[Base+$104];

VoicePos := (Temp0 SHL 7)+ (Temp1 SHR 8);

For I := 1 to 10 do GusDelay;

End;

Function  GUSPeek(Loc : Longint) : Byte;

Var

B : Byte;

AddLo : Word;

AddHi : Byte;

Begin

AddLo := Loc AND $FFFF;

AddHi := LongInt(Loc AND $FF0000) SHR 16;

Port [Base+$103] := $43;

Portw[Base+$104] := AddLo;

Port [Base+$103] := $44;

Port [Base+$105] := AddHi;

B := Port[Base+$107];

GUSPeek := B;

End;

Procedure GUSPoke(Loc : Longint; B : Byte);

Var

AddLo : Word;

AddHi : Byte;

Begin

AddLo := Loc AND $FFFF;

AddHi := LongInt(Loc AND $FF0000) SHR 16;

{  Write('POKE  HI :', AddHi:5, '  LO : ', AddLo:5, '    ');}

Port [Base+$103] := $43;

Portw[Base+$104] := AddLo;

Port [Base+$103] := $44;

Port [Base+$105] := AddHi;

Port [Base+$107] := B;

{  Writeln(B:3);}

End;

Function GUSProbe : Boolean;

Var

B : Byte;

Begin

Port [Base+$103] := $4C;

Port [Base+$105] := 0;

GUSDelay;

GUSDelay;

Port [Base+$103] := $4C;

Port [Base+$105] := 1;

GUSPoke(0, $AA);

GUSPoke($100, $55);

B := GUSPeek(0);

{  Port [Base+$103] := $4C;

Port [Base+$105] := 0;}

{ Above bit disabled since it appears to prevent the GUS from accessing

it's memory correctly.. in some bizare way.... }

If B = $AA then GUSProbe := True else GUSProbe := False;

End;

Procedure GUSFind;

Var

I : Word;

Begin

for I := 1 to 8 do

Begin

Base := $200 + I*$10;

If GUSProbe then I := 8;

End;

If Base < $280 then

Write('Found your GUS at ', Hex(Base), ' ');

End;

Function  GUSFindMem : Longint;

{ Returns how much RAM is available on the GUS }

Var

I : Longint;

B : Byte;

Begin

GUSPoke($40000, $AA);

If GUSPeek($40000) <> $AA then I := $3FFFF

else

Begin

GUSPoke($80000, $AA);

If GUSPeek($80000) <> $AA then I := $8FFFF

else

Begin

GUSPoke($C0000, $AA);

If GUSPeek($C0000) <> $AA then I := $CFFFF

else I := $FFFFF;

End;

End;

GUSFindMem := I;

End;

Procedure GUSSetFreq( V : Byte; F : Word);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := 1;

Portw[Base+$104] := (F { DIV 19}); { actual frequency / 19.0579083837 }

End;

Procedure GUSVoiceControl( V, B : Byte);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $0;

Port [Base+$105] := B;

End;

Procedure GUSSetBalance( V, B : Byte);

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $C;

Port [Base+$105] := B;

End;

Procedure GUSSetVolume( Voi : Byte; Vol : Word);

Begin

Port [Base+$102] := Voi;

Port [Base+$102] := Voi;

Port [Base+$102] := Voi;

Port [Base+$103] := 9;

Portw[Base+$104] := Vol;  { 0-0ffffh, log ... not linear }

End;

Procedure GUSSetLoopMode( V : Byte);

Var

Temp : Byte;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $80;

Temp := Port[Base+$105];

Port [Base+$103] := 0;

Port [Base+$105] := (Temp AND $E7) OR Mode;

End;

Procedure GUSStopVoice( V : Byte);

Var

Temp : Byte;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $80;

Temp := Port[Base+$105];

Port [Base+$103] := 0;

Port [Base+$105] := (Temp AND $df) OR 3;

GUSDelay;

Port [Base+$103] := 0;

Port [Base+$105] := (Temp AND $df) OR 3;

End;

Procedure GUSPlayVoice( V, Mode : Byte;VBegin, VStart, VEnd : Longint);

Var

GUS_Register : Word;

Begin

Port [Base+$102] := V;

Port [Base+$102] := V;

Port [Base+$103] := $0A;

Portw[Base+$104] := (VBegin SHR 7) AND 8191;

Port [Base+$103] := $0B;

Portw[Base+$104] := (VBegin AND $127) SHL 8;

Port [Base+$103] := $02;

Portw[Base+$104] := (VStart SHR 7) AND 8191;

Port [Base+$103] := $03;

Portw[Base+$104] := (VStart AND $127) SHL 8;

Port [Base+$103] := $04;

Portw[Base+$104] := ((VEnd)   SHR 7) AND 8191;

Port [Base+$103] := $05;

Portw[Base+$104] := ((VEnd)   AND $127) SHL 8;

Port [Base+$103] := $0;

Port [Base+$105] := Mode;

{ The below part isn't mentioned as necessary, but the card won't

play anything without it! }

Port[Base] := 1;

Port[Base+$103] := $4C;

Port[Base+$105] := 3;

end;

Procedure GUSReset;

Begin

port [Base+$103]   := $4C;

port [Base+$105] := 1;

GUSDelay;

port [Base+$103]   := $4C;

port [Base+$105] := 7;

port [Base+$103]   := $0E;

port [Base+$105] := (14 OR $0C0);

End;

Var

I : Longint;

F : File;

Buf : Array[1..20000] of Byte;

S : Word;

Begin

Clrscr;

Writeln('GUS DigiUnit V1.0');

Writeln('Copyright 1994 Mark Dixon.');

Writeln;

GUSFind;

Writeln('with ', GUSFindMem, ' bytes onboard.');

Writeln;

GUSReset;

End.

�����������������������������������������������������������������������������

� GUS669.PAS �

��������������

UNIT Gus669;

{

GUS669 Unit  v0.2b

Copyright 1994 Mark Dixon.

This product is "Learnware".

All contents of this archive, including source and executables, are the

intellectual property of the author, Mark Dixon. Use of this product for

commercial programs, or commercial gain in ANY way, is illegal. Private

use, or non-commercial use (such as demos, PD games, etc) is allowed,

provided you give credit to the author for these routines.

Feel free to make any modifications to these routines, but I would

appreciate it if you sent me these modifications, so that I can include

them in the next version of the Gus669 Unit.

If you wish to use these routines for commercial purposes, then you will

need a special agreement. Please contact me, Mark Dixon, and we can work

something out.

What's "Learnware"? Well, I think I just made it up actually. What i'm

getting at is that the source code is provided for LEARNING purposes only.

I'd get really angry if someone ripped off my work and tried to make out

that they wrote a mod player.

Beta version? Yes, since the product is still slightly unstable, I feel

it is right to keep it under beta status until I find and fix a few

bugs.

FEATURES

- Only works with the GUS!

- 8 channel, 669 music format.

- That's about it really.

- Oh, 100% Pascal high level source code = NO ASSEMBLER!

(So if you want to learn about how to write your own MOD player, this

should make it easier for you)

- Tested & compiled with Turbo Pascal v7.0

BUGS

- Not yet, give me a chance!

(If you find any, I would very much appreciate it if you could take

the time to notify me)

- Doesn't sound right with some modules, advice anyone??

- Could do with some better I/O handling routines when loading the

669 to give better feedback to the user about what went wrong

if the module didn't load.

You can contact me at any of the following :

FidoNet  : Mark Dixon  3:620/243

ItnerNet : markd@cairo.anu.edu.au         ( prefered )

d9404616@karajan.anu.edu.au    ( might not work for mail :) )

sdixonmj@cc.curtin.edu.au      ( Don't use this one often )

sdixonmj01@cc.curtin.edu.au    ( Might not exist any more,

that's how often it's used! )

I collect internet accounts.... :)

If you happen to live in the Australian Capital Territory, you can

call me on  231-2000, but at respectable hours please.

"Want more comments? Write em!"

Sorry, I just had to quote that. I'm not in the mood for writing lots

of comments just yet. The main reason for writing it in Pascal is so

that it would be easy to understand. Comments may (or may not) come later

on.

Okay, enough of me dribbling, here's the source your after!

}

Interface

Procedure Load669(N : String);

Procedure PlayMusic;

Procedure StopMusic;

Type

{ This is so that we can keep a record of what each channel is

currently doing, so that we can inc/dec the Frequency or volume,

or pan left/right, etc }

Channel_Type    = Record

Vol : Word;

Freq : Word;

Pan : Byte;

End;

Var

Channels : Array[1..8] of Channel_Type;

Flags : Array[0..15] of Byte;

{ Programmer flags. This will be explained when it is fully implemented. }

Const

Loaded : Boolean = False;    { Is a module loaded? }

Playing : Boolean = False;   { Is a module playing? }

WaitState : Boolean = False; { Set to TRUE whenever a new note is played }

{ Helpful for timing in with the player }

Const

NumChannels = 8;

{ Thanks to Tran for releasing the Hell demo source code, from which

I managed to find these very helpfull volume and frequency value

tables, without which this player would not have worked! }

voltbl : Array[0..15] of Byte =

(  $004,$0a0,$0b0,$0c0,$0c8,$0d0,$0d8,$0e0,

$0e4,$0e8,$0ec,$0f1,$0f4,$0f6,$0fa,$0ff);

freqtbl : Array[1..60] of Word = (

56,59,62,66,70,74,79,83,88,94,99,105,

112,118,125,133,141,149,158,167,177,188,199,211,

224,237,251,266,282,299,317,335,355,377,399,423,

448,475,503,532,564,598,634,671,711,754,798,846,

896,950,1006,1065,1129,1197,1268,1343,1423,1508,1597,1692 );

Type

Header_669_Type = Record

Marker      : Word;

Title       : Array[1..108] of Char;

NOS,                     { No of Samples  0 - 64 }

NOP         : Byte;      { No of Patterns 0 - 128 }

LoopOrder   : Byte;

Order       : Array[0..127] of Byte;

Tempo       : Array[0..127] of Byte;

Break       : Array[0..127] of Byte;

End;

Sample_Type     = Record

FileName  : Array[1..13] of Char;

Length    : Longint;

LoopStart : Longint;

LoopLen   : Longint;

End;

Sample_Pointer  = ^Sample_Type;

Note_Type       = Record

Info,  { <- Don't worry about this little bit here }

Note,

Sample,

Volume,

Command,

Data    : Byte;

End;

Event_Type      = Array[1..8] of Note_Type;

Pattern_Type    = Array[0..63] of Event_Type;

Pattern_Pointer = ^Pattern_Type;

Var

Header : Header_669_Type;

Samples : Array[0..64] of Sample_Pointer;

Patterns : Array[0..128] of Pattern_Pointer;

GusTable : Array[0..64] of Longint;

GusPos : Longint;

Speed : Byte;

Count : Word;

OldTimer : Procedure;

CurrentPat, CurrentEvent : Byte;

Implementation

Uses Dos, Crt, GUSUnit;

Procedure Load669(N : String);

Var

F : File;

I, J, K : Byte;

T : Array[1..8,1..3] of Byte;

Procedure LoadSample(No, Size : Longint);

Var

Buf : Array[1..1024] of Byte;

I : Longint;

J, K : Integer;

Begin

GusTable[No] := GusPos;

I := Size;

While I > 1024 do

Begin

BlockRead(F, Buf, SizeOf(Buf), J);

For K := 1 to J do GusPoke(GusPos+K-1, Buf[K] XOR 127);

Dec(I, J);

Inc(GusPos, J);

End;

BlockRead(F, Buf, I, J);

For K := 1 to J do GusPoke(GusPos+K-1, Buf[K] XOR 127);

Inc(GusPos, J);

End;

Begin

{$I-}

Assign(F, N);

Reset(F, 1);

BlockRead(F, Header, SizeOf(Header));

If Header.Marker = $6669 then

Begin

For I := 1 to Header.NOS do

Begin

New(Samples[I-1]);

BlockRead(F, Samples[I-1]^, SizeOf(Samples[I-1]^));

End;

For I := 0 to Header.NOP-1 do

Begin

New(Patterns[I]);

For J := 0 to 63 do

Begin

BlockRead(F, T, SizeOf(T));

For K := 1 to 8 do

Begin

Patterns[I]^[J,K].Info    := t[K,1];

Patterns[I]^[J,K].Note    := ( t[K,1] shr 2);

Patterns[I]^[J,K].Sample  := ((t[K,1] AND 3) SHL 4) +  (t[K,2] SHR 4);

Patterns[I]^[J,K].Volume  := ( t[K,2] AND 15);

Patterns[I]^[J,K].Command := ( t[K,3] shr 4);

Patterns[I]^[J,K].Data    := ( t[K,3] AND 15);

End;

End;

End;

For I := 1 to Header.NOS do

LoadSample(I-1, Samples[I-1]^.Length);

End;

Close(F);

{$I+}

If (IOResult <> 0) OR (Header.Marker <> $6669) then

Loaded := False else Loaded := True;

End;

Procedure UpDateNotes;

Var

I : Word;

Inst : Byte;

Note : Word;

Begin

WaitState := True;

For I := 1 to NumChannels do

With Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I] do

For I := 1 to NumChannels do

If (Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Info < $FE) then

Begin

Inst := Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Sample;

Note := Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Note;

Channels[I].Freq := FreqTbl[Note];

{    Channels[I].Pan  := (1-(I AND 1)) * 15;}

Channels[I].Vol  := $100*VolTbl[Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Volume];

{    Write(Note:3,Inst:3,' -');}

GUSSetVolume    (I, 0);

GUSVoiceControl (I, 1);

GUSSetBalance   (I, Channels[I].Pan);

GusSetFreq      ( I, Channels[I].Freq);

{    GUSPlayVoice    ( I, 0, GusTable[Inst],

GusTable[Inst],

GusTable[Inst]+Samples[Inst]^.Length  );}

{    Write(Samples[Inst]^.LoopLen:5);}

If Samples[Inst]^.LoopLen < 1048575 then

Begin

GUSPlayVoice    ( I, 8, GusTable[Inst],

GusTable[Inst]+Samples[Inst]^.LoopStart,

GusTable[Inst]+Samples[Inst]^.LoopLen  );

End

Else

Begin

GUSPlayVoice    ( I, 0, GusTable[Inst],

GusTable[Inst],

GusTable[Inst]+Samples[Inst]^.Length  );

End;

End;

{  Writeln;}

For I := 1 to NumChannels do

If (Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Info < $FF) then

GUSSetVolume (I, $100*VolTbl[Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I].Volume]);

For I := 1 to NumChannels do

With Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I] do

Case Command of

5 : Speed := Data;

3 : Begin

Channels[I].Freq := Channels[I].Freq + 10;

GUSSetFreq(I, Channels[I].Freq);

End;

8 : Inc(Flags[Data]);

6 : Case Data of

0 : If Channels[I].Pan > 0 then

Begin

Dec(Channels[I].Pan);

GusSetBalance(I, Channels[I].Pan);

End;

1 : If Channels[I].Pan < 15 then

Begin

Inc(Channels[I].Pan);

GusSetBalance(I, Channels[I].Pan);

End;

End;

End;

Inc(CurrentEvent);

If CurrentEvent > Header.Break[CurrentPat] then Begin CurrentEvent := 0; Inc(CurrentPat) End;

If Header.Order[CurrentPat] > (Header.NOP) then Begin CurrentEvent := 0; CurrentPat := 0; End;

End;

Procedure UpDateEffects;

Var

I : Word;

Begin

For I := 1 to 4 do

With Patterns[Header.Order[CurrentPat]]^[CurrentEvent, I] do

Begin

Case Command of

0 : Begin

Inc(Channels[I].Freq, Data);

GusSetFreq(I, Channels[I].Freq);

End;

1 : Begin

Dec(Channels[I].Freq, Data);

GusSetFreq(I, Channels[I].Freq);

End;

End;

End;

End;

{ $ F+,S-,W-}

Procedure ModInterrupt; Interrupt;

Begin

Inc(Count);

If Count = Speed then

Begin

UpDateNotes;

Count := 0;

End;

UpDateEffects;

If (Count MOD 27) = 1 then

Begin

inline ($9C);

OldTimer;

End;

Port[$20] := $20;

End;

{ $ F-,S+}

Procedure TimerSpeedup(Speed : Word);

Begin

Port[$43] := $36;

Port[$40] := Lo(Speed);

Port[$40] := Hi(Speed);

end;

Procedure PlayMusic;

Begin

If Loaded then

Begin

TimerSpeedUp( (1192755 DIV 32));

GetIntVec($8, Addr(OldTimer));

SetIntVec($8, Addr(ModInterrupt));

Speed := Header.Tempo[0];

Playing := True;

End

{ If the module is not loaded, then the Playing flag will not be set,

so your program should check the playing flag just after calling

PlayMusic to see if everything was okay. }

End;

Procedure StopMusic;

Var

I : Byte;

Begin

If Playing then

Begin

SetIntVec($8, Addr(OldTimer));

For I := 1 to NumChannels do GusSetVolume(I, 0);

End;

TimerSpeedUp($FFFF);

End;

Procedure Init;

Var

I : Byte;

Begin

GusPos := 1;

Count := 0;

Speed := 6;

CurrentPat := 0;

CurrentEvent := 0;

For I := 1 to NumChannels do Channels[I].Pan  := (1-(I AND 1)) * 15;

For I := 1 to NumChannels do GUSVoiceControl(I, 1);

For I := 0 to 15 do Flags[I] := 0;

End;

Var

I, J : Byte;

Begin

Init;

Writeln('GUS669 Unit V0.2b');

Writeln('Copyright 1994 Mark Dixon.');

Writeln;

End.

�����������������������������������������������������������������������������

� PLAY669.PAS �

���������������

Program Testout_Gus669_Unit;

Uses Crt, GUS669;

Begin

If ParamCount > 0 then Load669(Paramstr(1))

else

Begin

Writeln;

Writeln('Please specify the name of the 669 module you wish to play');

Writeln('from the command line.');

Writeln;

Writeln('eg :    Play669  Hardwired.669 ');

Writeln;

Halt(1);

End;

PlayMusic;

If Playing then

Begin

Writeln('Playing ', ParamStr(1) );

Writeln('Press any key to stop and return to DOS.');

Repeat

Until Keypressed

End

else

Begin

Writeln;

Writeln('Couldn''t load or play the module for some reason!');

Writeln;

Writeln('Please check your GUS is working correctly, and that you have');

Writeln('correctly specified the 669 filename.');

Writeln;

End;

StopMusic;

End.